Classified Raster Details used in the DSS-WRM

1. Land Use / Land Cover (LULC)

LULC is used in the following modules:

- STP Priority

- STP Suitability

- MAR Site Suitability

- GW Potential Zone

Membership Function: Linear

LULC Reclassification for STP Priority

| S. No. | LULC Class | Class Value | Rank | Suitability/Priority Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Water | — | 1 | Lowest priority for STP development |
| 2 | Trees | — | 2 | Low priority due to natural/vegetated land |
| 3 | Bare Ground | — | 3 | Moderate priority |
| 4 | Rangeland | — | 4 | Moderately high priority |
| 5 | Cropland | — | 5 | High priority due to potential wastewater/environmental concerns |
| 6 | Builtup Area | — | 6 | Highest priority due to high wastewater generation |

Ranking: Water < Trees < Bare Ground < Rangeland < Cropland < Builtup Area

LULC Reclassification for STP Suitability

| S. No. | LULC Class | Rank | Suitability Interpretation |
| --- | --- | --- | --- |
| 1 | Water | 1 | Unsuitable for STP development |
| 2 | Builtup Area | 2 | Low suitability due to land-use conflict |
| 3 | Trees | 3 | Low–moderate suitability due to environmental sensitivity |
| 4 | Cropland | 4 | Moderate suitability |
| 5 | Rangeland | 5 | Relatively high suitability |
| 6 | Bare Ground | 6 | Highest suitability from LULC perspective |

Ranking: Water < Builtup Area < Trees < Cropland < Rangeland < Bare Ground

LULC Reclassification for MAR Site Suitability

| S. No. | LULC Class | Rank | Suitability Interpretation |
| --- | --- | --- | --- |
| 1 | Builtup Area | 1 | Very low suitability because of impervious surfaces and land-use conflict |
| 2 | Water | 2 | Low suitability for direct MAR-structure development |
| 3 | Trees | 3 | Moderate suitability; environmentally sensitive areas should be considered carefully |
| 4 | Rangeland | 4 | Moderately high suitability |
| 5 | Cropland | 5 | High suitability for recharge interventions |
| 6 | Bare Ground | 6 | Highest suitability due to availability of open land |

Ranking: Builtup Area < Water < Trees < Rangeland < Cropland < Bare Ground

LULC Reclassification for GW Potential Zone

| S. No. | LULC Class | Rank | Groundwater Potential Interpretation |
| --- | --- | --- | --- |
| 1 | Builtup Area | 1 | Very low potential because impervious surfaces restrict infiltration |
| 2 | Bare Ground | 2 | Low–moderate potential; strongly dependent on soil and geology |
| 3 | Water | 3 | Moderate groundwater interaction but not necessarily groundwater-storage potential |
| 4 | Rangeland | 4 | Moderately high recharge potential |
| 5 | Cropland | 5 | High recharge potential |
| 6 | Trees | 6 | Highest natural infiltration/recharge contribution |

Ranking: Builtup Area < Bare Ground < Water < Rangeland < Cropland < Trees

Consolidated LULC Ranking Table

| LULC Class | STP Priority | STP Suitability | MAR Suitability | GW Potential Zone |
| --- | --- | --- | --- | --- |
| Water | 1 | 1 | 2 | 3 |
| Trees | 2 | 3 | 3 | 6 |
| Cropland | 5 | 4 | 5 | 5 |
| Rangeland | 4 | 5 | 4 | 4 |
| Bare Ground | 3 | 6 | 6 | 2 |
| Builtup Area | 6 | 2 | 1 | 1 |

2. Soil Texture

Soil Texture is used in:

- STP Suitability

- MAR Site Suitability

- GW Potential Zone

Your available classes are:

| Class Value | Soil Texture |
| --- | --- |
| 0 | Sand |
| 4 | Silt |
| 6 | Sandy Clay Loam |
| 7 | Clay Loam |

The class values above are the original raster/category values; the rank is the reclassified value used in the suitability analysis.

Soil Texture Reclassification for STP Suitability

For STP suitability, relatively finer-textured soils are preferred because they generally have lower permeability and can reduce rapid downward movement of wastewater/effluent.

| S. No. | Soil Texture | Original Class Value | Rank | Suitability Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Sand | 0 | 1 | Lowest suitability due to high permeability |
| 2 | Silt | 4 | 2 | Low–moderate suitability |
| 3 | Sandy Clay Loam | 6 | 3 | High suitability |
| 4 | Clay Loam | 7 | 4 | Highest suitability due to relatively low permeability |

Ranking: Sand < Silt < Sandy Clay Loam < Clay Loam

Soil Texture Reclassification for MAR Site Suitability

For MAR, higher infiltration capacity is generally desirable because the purpose is to facilitate recharge into the aquifer.

| S. No. | Soil Texture | Original Class Value | Rank | MAR Suitability Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Clay Loam | 7 | 1 | Lowest infiltration suitability |
| 2 | Silt | 4 | 2 | Low–moderate infiltration suitability |
| 3 | Sandy Clay Loam | 6 | 3 | High infiltration suitability |
| 4 | Sand | 0 | 4 | Highest infiltration suitability |

Ranking: Clay Loam < Silt < Sandy Clay Loam < Sand

Soil Texture Reclassification for GW Potential Zone

For groundwater-potential assessment, soils with greater infiltration capacity generally receive higher ranks because they facilitate recharge to the subsurface.

| S. No. | Soil Texture | Original Class Value | Rank | Groundwater Potential Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Clay Loam | 7 | 1 | Low infiltration and recharge potential |
| 2 | Silt | 4 | 2 | Low–moderate recharge potential |
| 3 | Sandy Clay Loam | 6 | 3 | High recharge potential |
| 4 | Sand | 0 | 4 | Highest infiltration/recharge potential |

Ranking: Clay Loam < Silt < Sandy Clay Loam < Sand

Consolidated Soil Texture Ranking Table

| Soil Texture | Original Class Value | STP Suitability | MAR Suitability | GW Potential Zone |
| --- | --- | --- | --- | --- |
| Sand | 0 | 1 | 4 | 4 |
| Silt | 4 | 2 | 2 | 2 |
| Sandy Clay Loam | 6 | 3 | 3 | 3 |
| Clay Loam | 7 | 4 | 1 | 1 |

3. Geomorphology

Geomorphology is used in:

- STP Suitability

- GW Potential Zone

The available geomorphological classes are:

| Class Value | Geomorphology Class |
| --- | --- |
| 1 | Older Alluvial Plain |
| 2 | Older Flood Plain |
| 3 | Pond |
| 4 | River |
| 5 | Water Body–Lake |
| 6 | Water Body–Unclassified |

The module document specifies Geomorphology as a Linear factor in both STP Suitability and GW Potential Zone.

Geomorphology Reclassification for STP Suitability

For STP development, stable terrestrial geomorphic units are preferred, while water bodies and active water environments are least suitable.

| S. No. | Geomorphology Class | Original Class Value | Rank | STP Suitability Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Water Body–Lake | 5 | 1 | Very low suitability |
| 2 | Water Body–Unclassified | 6 | 1 | Very low suitability |
| 3 | River | 4 | 2 | Very low suitability due to direct surface-water interaction |
| 4 | Pond | 3 | 3 | Low suitability |
| 5 | Older Flood Plain | 2 | 4 | Moderate suitability |
| 6 | Older Alluvial Plain | 1 | 6 | Highest suitability |

Ranking: Water Body–Lake/Unclassified < River < Pond < Older Flood Plain < Older Alluvial Plain

Important: The two water-body classes have been assigned the same lowest rank because both represent areas that should be avoided for STP development. Your separate Waterbody constraint in the STP Suitability model should ultimately exclude these areas.

Geomorphology Reclassification for GW Potential Zone

For groundwater potential, alluvial and flood-plain environments are generally favorable because of their association with unconsolidated sediments and groundwater storage/recharge conditions.

| S. No. | Geomorphology Class | Original Class Value | Rank | Groundwater Potential Interpretation |
| --- | --- | --- | --- | --- |
| 1 | Water Body–Unclassified | 6 | 1 | Very low as a groundwater-potential land unit |
| 2 | Water Body–Lake | 5 | 2 | Low |
| 3 | Pond | 3 | 3 | Moderate |
| 4 | River | 4 | 4 | Moderate–high groundwater interaction |
| 5 | Older Flood Plain | 2 | 5 | High groundwater potential |
| 6 | Older Alluvial Plain | 1 | 6 | Very high groundwater-storage potential |

Ranking: Water Body–Unclassified < Water Body–Lake < Pond < River < Older Flood Plain < Older Alluvial Plain

Consolidated Geomorphology Ranking Table

| Geomorphology Class | Original Class Value | STP Suitability | GW Potential Zone |
| --- | --- | --- | --- |
| Older Alluvial Plain | 1 | 6 | 6 |
| Older Flood Plain | 2 | 4 | 5 |
| Pond | 3 | 3 | 3 |
| River | 4 | 2 | 4 |
| Water Body–Lake | 5 | 1 | 2 |
| Water Body–Unclassified | 6 | 1 | 1 |